EE299 Project Course

Vehicular Ad Hoc Networks (VANET)

Advisor: Professor Izhak Rubin

Felix Lu

Contents

Routing in Sparse Vehicular Ad Hoc Wireless Networks	4

Authors: Nawaporn Wisitpongphan, Fan Bai, Priyantha Mudalige, Varsha Sadekar, and Ozan Tonguz	4

IEEE Journal on Selected Areas in Communications, Vol. 25, No. 8, October 2007 (pg 1538-1556)	4

Introduction of Routing in Sparse VANET	4

Describe Problem	4

Proposed Model	5

Empirical Research Results	6

Solution	8

Characteristics of Disconnected Vehicular Ad Hoc Network	9

Analysis	11

Analysis of Disconnected networks with Two Directional Traffic	12

Re-healing time Analysis	14

Simulations and Results	18

Performance Results	21

Discuss Results	21

Suggestion	21

Vehicular Mobility Simulation for VANETs	22

Authors: Marco Fiore, Jerome Harri, Fethi Filali, Christian Bonnet	22

Simulation Symposium, 2007. ANSS ’07. 40th Annual	22

Describe Problem	22

Solution	22

VanetMobiSim	23

Simulation	29

Performance Results	30

FTM (Fluid Traffic Model) Results	30

IDM (Intelligent Driver Model)	31

IDM-IM Stops and Lights (Intelligent Driver Model with Intersection Modeling)	32

IDM-LC Lights (Intelligent Driver Model	33

Possible Improvements on the Model	34

Conclusion	34

Analytical Model for Connectivity in Vehicular Ad Hoc Networks	35

Authors: Saleh Yousefi, Eitan Altman, Rachid El-Azouzi, Mahmood Fathy	35

IEEE Transactions on Vehicular Technology, Vol. 57, No. 6, November 2008	35

Describe Problem	35

Analytical Model	36

Distribution of Inter-vehicle Distances	37

Connectivity Analysis	39

Simulation and Analysis	41

Varying Traffic Parameters	43

Performance Results	43

Discuss Results	43

Suggestions	43

References	44

[1] Routing in Sparse Vehicular Ad Hoc Wireless Networks	44

[2] Vehicular Mobility Simulation for VANETs	44

[3] Analytical Model for Connectivity in Vehicular Ad Hoc Networks	44

# Routing in Sparse Vehicular Ad Hoc Wireless Networks

# Authors: Nawaporn Wisitpongphan, Fan Bai, Priyantha Mudalige, Varsha Sadekar, and Ozan Tonguz

# IEEE Journal on Selected Areas in Communications, Vol. 25, No. 8, October 2007 (pg 1538-1556)

## Introduction of Routing in Sparse VANET

This paper focuses on a specific case of a Vehicular Ad Hoc Network (VANET) where a fully connected network infrastructure may not be present.  In a real world setting, the availability of relay towers or other vehicles capable of relaying information may not be in range or not present at all. In the article “Routing in Sparse Vehicular Ad Hoc Wireless Networks”, the assumption is that a communication network will be fragmented and sparse at times and the models and frameworks developed will take into account the varying vehicle densities and market penetration of a given network system. By using this assumption the models developed to describe network behavior given varying vehicular densities are proven to be acceptably accurate by comparing the analytical results against multiple Monte Carlo simulations as well as data collected from empirical studies of real world traffic scenarios.

## Describe Problem

Much research is devoted to developing routing algorithms and protocols for communication between vehicles in a dense network setting or resolving the issue of broadcast storms. Although this is a valid scenario it does not include scenarios where the connectivity of a network may not be reliable such as in rural areas or late night traffic situations where it may not be possible to maintain a connected network. The research set forth will instead focus on the situation where a consistent connected network is not always feasible and develop models that incorporate network fragmentation into the solution.

Focusing on the fact that any particular VANET will at times experience network discontinuity three key questions must be answered:

- What are the consequences when trying to relay safety messages to other vehicles when the network is constantly disconnected?

- In a disconnected VANET what are the key characteristics to observe and what are the adverse effects on performance in the network?

- What is the solution to a disconnected VANET if any? And if there is not a viable solution to this problem how can the affects of network fragmentation become minimized?

In this paper, not only will it highlight the reality of network fragmentation in real world settings, it will also introduce a possible solution to the disconnected network issue.  The solution proposed, “Stored-carry-forward” mechanism, will prove to be viable through analytical and simulation results, as well as comparing with empirical data collected by the Berkeley Highway Laboratory (BHL).

The BHL research is vital to proposed model because it realistically influences important parameters of the model such as inter-arrival time and inter-vehicle spacing between vehicles.  For example through the empirical studies the results showed that inter-vehicle spacing follows an exponential distribution when the traffic volume is below 1000 vehicles/hour. Furthermore, with the assumption that all vehicles possess the technology of wireless communications, the network still experienced network discontinuity of up to 35% during hours of low traffic volume.

## Proposed Model

The foundation of this model is based on a well known model, the Car following model. This model assumes a single-lane scenario with any type of traffic conditions. Another assumption is that drivers drive safely and that drivers’ deceleration factors are taken into consideration. The model is shown below:

| S'= L+ β'V+ γV2 | - S’ - headway spacing between rear bumper to rear bumper - L - effective vehicle length in meters - β' - driver reaction time in seconds - V - vehicle sped in meters/second - γ - reciprocal of twice the max average deceleration of a following vehicle (ie approximately 0.075 s2/m) |
| --- | --- |

To simplify the model, γ≈0 with the “good-driver” assumption. This assumes that every driver will have a similar deceleration factor which simplifies the above the original equation to:

S'= L+ β'V

The car following model describes adjacent vehicles that are travelling in the same lane. From a network perspective, it is irrelevant whether the following car is in the same lane or next because our focus is network connectivity, which depends more on the distance away from a particular vehicle regardless of position. Because of this the model is expanded to represent a multi-lane system.  Below we observe the distinction between a single-lane (blue arrows) and multi-lane (red-arrows) model. The arrows in blue are only associated with vehicles directly in the same lane where as the red arrows are associated with the next closest vehicle in proximity.

Another adjustment that needs to be made is with β' (the driver reaction time). Through raw data collection during moderate to light traffic, β' was as large as 50-100 seconds. This is unrealistically high as a representation for driver reaction time, so instead β will replace β' as the inter-arrival time of vehicles. The final adjustment is the minimum spacing L. Because of the multi-lane configuration we expand L to Lmin to represent the minimum distance between any 2 vehicles regardless if they are in the same lane (red arrows). The new model then becomes:

| S= Lmin+ βV | - S – road level inter-vehicle spacing - Lmin - minimum spacing between any two adjacent vehicles - β - inter-arrival time of vehicles on any lane from fix observation point - V - vehicle sped in meters/second |
| --- | --- |

This model is now capable of characterizing traffic conditions in both heavy and light traffic situations. The next step to making this model more tangible is to correctly or approximately calculate the inter-arrival time and vehicle speeds.  We can leverage the data collected by BHL to approximate these two parameters to give realistic values for β and V in different traffic situations.

## Empirical Research Results

The basis of the proposed research solution is based heavily on empirical data collected in a real world environment. The data was collected by BHL on the eastbound I-80 five lane freeway on June 27, 2006 for a time span of 24-hours with a recording frequency of 60 recordings per second.  Given the measurements it is possible to approximate the inter-arrival times and average vehicle speeds for different traffic conditions. To better represent vehicular traffic conditions three different time periods are used to distinctly represent three varying conditions. 1am-3am models sparse traffic conditions where vehicles are traveling at higher speeds, 10am-12pm represents moderate traffic conditions and average speed, and 3pm-5pm will be the case of rush hour where vehicles are tightly packed traveling at low speeds. Figure 2 and 3 are the results charting the vehicles/hour (traffic density) and speed of vehicles against the time of day.

From the data in figure 2 and 3 the inter-arrival time β and the average vehicle speed V can be approximated for the three different vehicle density scenarios. Graphing the results of the extrapolation we can see that the inter-arrival time β can be closely approximated by an exponential distribution during the early morning period while the speed of vehicles resembles a normal distribution.  Because this research is centered on the notion that a network will inevitably be fragmented we will emphasize the results between 1am-3am which most closely represents a network that will display sporadic connective behavior. Figure 4 shows the Probability Density Functions (PDF) of both the inter-arrival times and vehicle speeds. Particularly figure 4(a) will be of high value in aiding to determine an appropriate inter-arrival time β.

## Solution

65% chance of being in range (R ≤ 250 meters)

First we must define what is meant by disconnection from the network. We will consider two vehicles to be network disconnected if their distance is greater than 250 meters. Given this constraint we need to develop an expression for the inter-arrival time β . The rate of measuring the inter-arrival time was 60 recordings per second so we can characterize the parameter λt ≈Tv3600 to fit the collected data of volume per hour. From figure 4(a) for the time period of 1-3am (the scenario that we are concerned with) we observed earlier that it fits an exponential distribution so we can characterize the PDF of β by fββ= λte-λtβ (Note: the general form of an exponential distribution is fxx=λe-λx). Earlier we derived the inter-vehicle spacing equation S= Lmin+ βV. Given the empirical data collected a reasonable approximation for the inter-vehicle spacing S is simply the product of the inter-vehicle arrival time and vehicle speeds (S ~ βV). Having a PDF for S we can then calculate the cumulative density function (CDF) for the inter-vehicle spacing. Figure 5 shows the empirical results for the PDF and CDF of the inter-vehicle spacing.

First looking at figure 5(b), we can see that the PDF of S for the time frame of 1-3am shows that the inter-vehicle spacing varied from 0 – 1000km. By taking the integration of the PDF fS(S) in which the inter-vehicle spacing is less than R=250m, then we can gauge the frequency of being disconnected in the network. By solving FSR=PS< R= 0RfS(S) and plotting the CDF we are able to derive figure 5(b). For R=250m we can visually see that there is about a 65% chance of being within range of the communication network, meaning we have a 35% chance of being disconnected network. However, in reality the probability of being disconnected will be higher because this result assumes that every vehicle possesses the equipment to be connected in the network.

In a similar vein we can represent the Inter-vehicle spacing with a similar exponential distribution. First we define the parameter λs ≈Tv3600 × V=λtV. Now we can then define the Inter-vehicle spacing to be fSs= λSe-λSs. Later on we will analyze how similar the two derived PDF are with the empirical findings for the time frame 1-3am, which suits the basis of our study of a randomly disjointed network.

### Characteristics of Disconnected Vehicular Ad Hoc Network

As stated before the chosen time frame of study is between 1-3am because it represents the case when a VANET is periodically disjointed. To properly model this scenario many factors have to be taken into consideration. Specifically the goal is to develop a model to also characterize the “probability of being the last vehicle in the cluster, average intra-cluster spacing, average inter-cluster spacing, average cluster size, and average cluster length [1].”

Going back to the minimum distance for network connection, R≤250m, a cluster is defined as any adjacent vehicles that are within the range of R.  Studying cluster characteristics will shed light on a disconnected VANET as well as aid in developing the framework to model such a VANET.

#### Probability of Being the Last Vehicle in a Cluster

We want to know the probability of being the last vehicle in a cluster because this should be the last vehicle to disseminate a message across a cluster boundary to optimize the end-to-end latency. We can make use of the inter-vehicle spacing PDF by calculating the probability of a distance greater than R=250m.

Inter-Vehicle Spacing PDF fSs= λSe-λSs

FSR=PS< R= 0RfSS
FSR= -e-λSs|0R=1- e-λSR

Now we can calculate the probability of being the last vehicle in a cluster, in other words the probability of having an Inter-Vehicle spacing greater than R=250m.

Pd=Prs>R
=1-Prs≤R
=1- FSR
Pd=1- 1- e-λSR= e-λSR

#### Average Intra-cluster Spacing

The Intra-Cluster spacing is described as the vehicle spacing within a given cluster. Vehicles within the proximity of R≤250 are considered to be in the same cluster, thus the PDF of the Intra-Cluster spacing can be defined as follows:

fSintraSintra=PrSS< R= λse-λsSintra1- e-λsR

Taking the expectation of fSintra we get:

ESintra=0RSintra ×fSintraSintra
=0RSintra × λse-λsSintra1- e-λsR
Using integration by parts
= -Sintra×e-λsSintra1- e-λsR- 1λSintrae-λsSintra1- e-λsR|0R 
ESintra= 1λs- Re-λsR1- e-λsR

#### Average Inter-Cluster Spacing

The Inter-Cluster spacing is the shortest distance between two adjacent clusters. This is most easily visualized as the distance between the last vehicles in a cluster with the first vehicle in a separate following cluster. The inter-cluster spacing is the inter-vehicle spacing given that the distance is greater than R: fSinterSInter=PrSS>R= λse-λs(sInter-R)

In a similar fashion we can take the expectation to get the expected Inter-cluster spacing:

ESinter= R∞Sinter × fSinterSInter 
=R∞Sinter × λse-λssInter-R 
Using integration by parts
=Sintere-λssInter-R- 1λS e-sInter-R|R∞

ESinter=R+ 1λs

#### Average Cluster Size

The average cluster size is also of importance. The method used to determine the average cluster size is to list the possibilities of the number of vehicles in a cluster before becoming disjointed. This forms a geometric series where the Probability Mass Function (PMF) can be calculated as follows:

fCN1= PrS1>R                                   = Pd
fCN2= PrS1≤R & S2>R                 = Pd(1-Pd)
fCN3= PrS1≤R& S2≤R & S3>R= Pd(1-Pd)2
fCNcn= Pd(1- Pd)cn-1

We can recognize that the expectation of a standard geometric series of this form will lead us to the average cluster sizeECN= 1Pd.

#### Average Cluster Length

The average cluster length can be derived from a combination of the cluster size and intra-vehicle spacing.

ECL=1Pd-11λs- Re-λsR1- e-λsR

## Analysis

The method used to measure the accuracy of the newly developed model against the empirical data is the Kolmogorov-Smirnov test (K-S test). This is a statistical test to compare the accuracy of sample data versus a reference probability distribution. In this case the K-S test will compare the empirical data collected by BHL and the derived CDF for the Inter-Arrival Time and Inter-Vehicle Spacing.  Using the K-S Test we can measure the goodness-of-fit by:

| Dn+,Dn- =max{Fnx- Fx, Fx- Fnx} | - Fx is from the model - Fnx is from the empirical data |
| --- | --- |

In figure 6 we can see the results of the K-S test. The model is accurate to about 3% for both Inter-arrival time and Inter-vehicle spacing during the time period of 1-3am. We observe that the dashed blue lines (model) and the solid blue line (empirical) are very similar. However the model breaks down slightly when vehicles exceed 1000 vehicles/hour. This is not too much of a concern mainly because we are focusing on the time frame when network connectivity is not guaranteed (i.e. 1-3am). Table I and II show the exact D statistics calculated with the K-S test to further reinforce the illustration in figure 6.

| Table I Kolmogorov-Smirnov Test Results of Inter-Arrival Time During Three Different Time Periods |  |  |  |
| --- | --- | --- | --- |
| Time of Day | Tv [veh/hr] | λt[veh/s] | (Dn+, Dn- ) [%] |
| 01:00 am – 03:00 am | 429 | 0.1192 | (2.89, 2.00) |
| 10:00 am – 12:00 pm | 2619 | 0.7276 | (4.65, 4.00) |
| 15:00 pm – 17:00 pm | 2812 | 0.7813 | (8.23, 10.98) |

| Table II Kolmogorov-Smirnov Test Results of Inter-Vehicle Spacing During Three Different Time Periods |  |  |  |  |
| --- | --- | --- | --- | --- |
| Time of Day | V[m/s] | λt[veh/s] | λs[veh/m] | (Dn+, Dn- ) [%] |
| 01:00 am – 03:00 am | 30.93 | 0.1192 | 0.0039 | (2.65, 2.42) |
| 10:00 am – 12:00 pm | 29.15 | 0.7276 | 0.0250 | (3.13, 9.27) |
| 15:00 pm – 17:00 pm | 10.73 | 0.7813 | 0.0728 | (8.09, 9.67) |

### Analysis of Disconnected networks with Two Directional Traffic

With traditional routing schemes it might not be a trivial task to communicate between separate clusters.  Vehicles following a broadcasting vehicle may be well out of range and would never receive any messages. In a VANET topology the use of opposite flowing traffic can be utilized to relay messages. As depicted in the figure 8, vehicle Src has a message pertaining to an accident that it needs to transmit to vehicle Dst. However if both are traveling at the same velocity, then it is possible that Dst will constantly be out of transmission range of Src. In the VANET topology vehicle Src may pass the message to either vehicle X, Y, or Z as they pass in the opposite direction, and then they in turn can relay the message to vehicle Dst.

In the scenario where no vehicles are in range of Src in the opposite direction, the message must be stored for later transmission when it detects a vehicle within range R. This “store-carry-forward” is referred to as temporal relay as there must be some time to wait for another node to be in communication range. Any communication where other nodes are immediately within range will further be referred to as spatial relay. To better characterize the effects of transmitting to vehicles in heading in the opposite direction we must assess the end-to-end packet delay and the number of spatial and temporal relays.

The first characteristic to capture involving the opposite traffic is the probability of being disconnected. If we let the inter-arrival spacing in the opposite direction have a parameter of λothen the PDF of the inter-vehicle spacing in the opposite direction is defined as:

fos= λoe-λos

Because we are dealing with opposite facing traffic, the range of disconnection is now 2R. It is possible to be disconnected before and after the transmitting vehicle as seen in the following illustration.

Given the range of 2R we can now solve for the CDF to find the probability of being disconnected from vehicles traveling in the opposite direction.

Fo2R=PS< 2R= 02RfoS
Fo2R= -e-λos|02R=1- e-λo2R

Let us define the probability of being disconnected in the opposite direction as Pd', then its definition is defined as the probability that the inter-vehicle spacing is greater than the range of 2R.

Pd'=Prs>2R
=1-Prs≤2R
=1- Fo2R
Pd'=1- 1- e-λo2R= e-λo2R

Now that we defined the probability of being disconnected in the opposite direction, and from earlier we derived the probability of being disconnected in the same direction, the probability of being disconnected in both directions is a product of the two probability spaces. Let us define this probability by Pd2.

Pd2=Pd×Pd'
=e-λSR ×e-λo2R
Pd2=e-(2Rλo+ Rλs)

### Re-healing time Analysis

To properly analyze the delay metrics of this VANET scheme we must capture the delays incurred from spatial and temporal relays.  The temporal relay is when a message is transmitted across clusters to reach its destination. To describe this delay we introduce the re-healing time, Tr. This is crucial in calculating the end-to-end delay to transmit a message because it is the bulk of the delay times. The spatial delay is the delay incurred when transmitting messages within the same cluster. For simplicity the spatial delays are considered negligible. This would make the end-to-end delay, Dtr, the sum of the re-healing times from the message transmission to the destination.

To facilitate the re-healing time analysis we first define the scenario to analyze. Travelling eastbound will be two vehicles Src and Dst, the source vehicle to transmit a message and the destination vehicle as the intended receiver of the message respectively. Vehicle Dst is out of range to receive messages from Dst and both are assumed to be travelling at the same velocity. To describe the traffic conditions, velocities and traffic densities eastbound are denoted with a subscript ‘e’ (λe, ve) and westbound are denoted with a subscript ‘w’ (λw, vw).

#### Best Case Scenario

The best scenario, as depicted in figure 9, is the case when a relay vehicle, Z, is immediately in the range of the transmitting vehicle Src. Vehicle Z and its associated cluster are also travelling in the opposite direction of the transmitting vehicle Src, in this setup westbound. Given this best case scenario the relaying cluster is immediately within range of vehicle Dst or not.

First there are two assumptions to be made: 1) Let the lane separation between vehicle Src and the relaying cluster be negligible. 2) Let vehicle Src be located in the middle of the relaying cluster. In figure 9 (a) this depicts the scenario when the difference between the inter-vehicle spacing and radius R is greater than half the relaying cluster’s length. The difference of (Sinter-R) is the distance needed to reach the range of vehicle Dst and CL is the cluster length. The probability of this scenario occurring is then:

p=Pr[CL2≤ SInter-R]

Earlier the cluster number and average intra-vehicle spacing was derived which can be used to approximate this equation. The cluster length is approximately the product of the number of vehicles in a cluster and the average intra-vehicle spacing. This leads to:

p=PrCN×ESintra2≤ESinter-R
≈CN≤ 2ESinter- RESintra
≈PrCN≤k
p≈1-(1- e-λwR)k, where k= 2λw(1- e-λwR)λe- λee-λwR(1+Rλw)

Given this probability of having a vehicle Z in range of vehicle Src, then the conditional re-healing time incurred from relaying this message is the time it takes for the first vehicle in Z’s cluster to come within the range of R of vehicle Dst. This can be expressed as follows:

ETr0≈ ESinter- R-ECL2CL2≤Sinter-Rve+vw
≈1λe-E[Sinter]2E[CN|CN≤k]ve+vw, where ECNCN≤k=1-k+1[FSwR]k+FSwR-[FSwR]k+11-FSw(R)

In the event that the cluster size is only 1 we must handle the situation differently. If instead in figure 9(a) the cluster was only comprised of vehicle Z, when it reaches in range of vehicle Dst at that instant its distance from the next receiving vehicle, say Dst2, is E[Sinter]+R. The message is immediately transferred from vehicle Dst back to Z en route to Dst2. In this scenario the re-healing time can be approximated by:

ETr1≈ R+ESinter- Rve+vw
≈Rλe+1λe(ve+vw)

For there to be another cluster to reach the distance between the relay and receiving vehicle must be greater than distance R. Recall that this constraint is given by the probability of being disconnected Pd. With this then the average re-healing time in the best case scenario becomes:

ETr1=1-Pd×ETr0+ Pd×ETr1
=1-e-λeRve+vw1λe-121λw-Re-λwR1-e-λwR×1-k+1(1-e-λwR)k+1-e-λwR-(1-e-λwR)k+1e-λwR+e-λeRRλe+1λe(ve+vw)×1-(1-e-λwR)k

#### Worst Case Scenario

The worst case scenario occurs when the vehicle Src has a message to transmit and there are no cars within the range R, both in the same and opposite directions. Figure 10 depicts this worst case scenario. The re-healing time now becomes the delay incurred while waiting for vehicle Z to arrive in its broadcast range summed with the delay for vehicle Z to be in range of vehicle Dst.

Temporal delay from Src to Z

The first case is when the Src vehicle is in a cluster where the length is less than the difference between the average inter-vehicle spacing and twice the range R ESinter'- 2R. Given this condition we can have at most M vehicles where M= ESinter'- 2RE[Sintra]. The probability of this occurring is p201=Pr2≤CN≤MCN>1= 1-(1-Pd)M-Pd1-Pd. Again for simplicity the assumption is that the spatial delay is negligible. With that the temporal delay for this case is approximately:

ETr201≈E[CL]ve+vw

The second case is when then vehicle Src is in the same cluster as the original transmitting vehicle, in other words a large cluster with greater than M vehicles (M as described in the first case). If this is the case then we know that Sinter' is greater than 2R, which then the re-healing time becomes the following with probability (1-p201)

ETr202≈0.5ESinter'Sinter'>2R-Rve+vw
≈12λw(ve+vw)

Combining the two scenarios we get the expected re-healing time from vehicles Src to Z to be:

ETr20≈p201× ETr201+ (1-p201) ETr202

Temporal delay from Z to Dst

The re-healing time from the relay vehicle Z to the destination Dst is the time when Z first receives the message until Z is within range R of vehicle Dst. We start the delay as soon as vehicle Z receives the message from vehicle Src, therefore the distance of vehicle Z from Dst at that instance in time is R+ESinter. This delay can denoted by:

ETr21≈R+ESinter- Rve+vw
≈Rλe+1λe(ve+vw)

The total re-healing time can now be characterized as the sum of the delay for vehicle Z to reach the range of Src and the delay from when Z reaches the range of vehicle Dst.

ETr2=ETr20+ETr21
= 1-(1-e-λeR)M-e-λeR(1-e-λeR)(ve+vw)×1e-λeR-11λe-Re-λeR1-e-λeR-12λw+12λw(ve+vw)+Rλe+1λe(ve+vw)

Where M=1-e-λeR(λeλw-Rλe)1-e-λeR-Re-λeR

## Simulations and Results

A series of Monte Carlo simulations were done in order to validate the developed framework. These simulations not only validate the proposed framework for a disconnected VANET, but are conversely validated by the empirical results as well. The setup for the simulation of the model is as follows:

Network Topology: Similar to the proposed model, the simulation will consist of a straight freeway with double sided traffic. The inter-vehicle spacing is given by the equation derived earlier, fSs= λSe-λSs. Given λS, the number of vehicles in the network can be approximated.

Mobility Pattern: Vehicles that enter the system will travel at a constant speed. This means the vehicle spacing between two vehicles will not change while they are in the system.  This also means that no vehicles will pass one another during the simulation. Any vehicle exiting the system will not be reintroduced as all vehicles entering the system will be newly generated.

Data Traffic Model: At any moment there is at most just 1 packet in the network. This packet will be generated anytime the Dst vehicle exits the network system. In this manner it is assured that every packet generated will have a unique network topology. The approximate rate of packet generation is about 0.01 packets/minute.

Network Communication Model: As in the development of the framework the range of communications for the devices is set to R=250m. The vehicles to represent Src and Dst will be chosen at random and their distance apart will be a random X km. Since the emphasis is on a disconnected VANET system the spatial relay times are assumed to be negligible and the temporal relay times will be tracked. This is a fair assumption since the temporal delays are magnitudes larger than delays incurred with MAC and Physical layers in network, so for simplicity this assumption is acceptable.

About 35% chance of being disconnected

In figure 11 we see the results of the probability of being disconnected, both for one way and bi-directional traffic. The introduction of vehicles into the system varied from 0.05 to 0.55 vehicles/second. As stated earlier vehicles traveled at a constant velocity. First note that the square boxes (analytical results) lie extremely close to the dashed lines (simulated results).  By varying the traffic in the model we clearly can see that the results from the developed framework to model network discontinuity is precise. We can see from the red line at about 500 vehicles/hour, which is approximately the rate of vehicles given the scenario from 1-3am, that there is a 35% chance of being disconnected from the network. Recall from the empirical data that the probability of being disconnected is also approximately 35%. Also the framework correctly models what we would intuitively think about traffic densities. As traffic volume increases the probability of being disconnected is lower and the vice versa is true. So the framework is validated both by the simulation and empirical results for modeling a disconnected VANET.

Figure 12 shows simulation results that further validate the accuracy of the framework developed. Looking at figures a and b, we can clearly deduce that as the traffic volume increases both the inter and intra-vehicle spacing decreases. Once again relying on intuition this makes logical sense because as more vehicles enter a freeway at a higher rate the distance between it and the preceding vehicle become smaller. This also bodes well for figures c and d as we see the average cluster length and size increase with traffic volume. This also makes sense because as traffic density increases more vehicles are within the range of R=250m which increases the number of vehicles in a cluster and adversely the cluster length.

The average per-gap healing time is intricate in developing the model. The purpose of understanding the characteristics in a disconnected VANET system was to develop a protocol to mitigate the inevitable network fragmentation and to analyze the re-healing time it would take to store-carry-forward a message to a destination vehicle. Looking at how vehicle speed impacts the average re-healing time in figure 15(a), it is clear that as vehicles increase their velocity the re-healing time decreases. This inverse relation is due to the speed in which opposite traffic can relay the messages thus decreasing the temporal delay.

Another scenario to decrease temporal delay, consequently reducing the re-healing time, is having an increased traffic volume such as in figure 15(b). Earlier the assumption was that spatial delay is negligible when compared to temporal delays. From figure 12 when traffic volume is increased we saw that the inter and intra-vehicle gaps decreased. Given this, there is a higher rate of spatial relays as we also saw that the average cluster lengths and sizes increased. What it ultimately comes down to is that as traffic volume increases, the density of each cluster increases, increasing the spatial relays and reducing the gaps between temporal delays. As a result the end-to-end delays dependent on temporal delays are also reduced. Again for both graphs in figure 15 we observe that the simulated results are particularly similar to the analytical results.

## Performance Results

## Discuss Results

## Suggestion

# Vehicular Mobility Simulation for VANETs

# Authors: Marco Fiore, Jerome Harri, Fethi Filali, Christian Bonnet

# Simulation Symposium, 2007. ANSS ’07. 40th Annual

## Describe Problem

With the advent of wireless technology it is inevitable that it eventually penetrate the world of vehicles.  One such example is the rapid development of VANETs.  VANETs are built with groups of moving vehicles, so developing protocols and testing in a live scenario is not practical or efficient.  Due to the nature of existing networking protocols and the grand scale it takes to test and develop this technology, it is vital to develop proper simulation models that can realistically characterize the nature of traffic conditions in today’s society.  Having models that accurately depict real world driving conditions greatly facilitate the development and eventual deployment of new VANET protocols that can greatly enhance everyone’s driving experience. What Fiore hopes to show is the development of an enhanced vehicle modeling system that can reliably model real world traffic conditions taking into account factors such as stop signs versus lights, typical speeds of roads, and even randomized individual behaviors.

Many current models only account for a few key characterizations of traffic such as distance to travel, average speeds, or average number of vehicles on a road.  Although these are valid it is not enough to model the true nature of traffic in today’s modern world.  Traffic is more complex as each driver is unique in their driving styles and reaction to different stimulations. Some drivers choose to stay in the same lanes while others feel the urge to weaver through traffic and pass anyone deemed as a barrier on the roads.  A key to this simulation is to model what is described as macro-mobility and micro-mobility descriptions which will hopefully better characterize traffic conditions, leading to better development of VANET protocols.

## Solution

First we begin with a brief description of what is included in macro and micro mobility.  Macro-mobility refers to driving conditions of the physical nature. In this simulation they include: “the road topology, constraining cars movement, the per-road characterization defining speed limits, number of lanes, overtaking and safety rules over each street, or the traffic signs description establishing the intersections crossing rules.” Micro-mobility refers to characteristics of the drivers. This is important because some existing models do not take this into account, but it definitely is a factory in everyday traffic. It includes some of the following: “traveling speed in different traffic conditions, acceleration, deceleration and overtaking criteria, behavior in presence of road intersections and traffic signs, general driving attitude related to driver’s age, sex, mood, etc.”

The simulations ran make use of an open source program called VanetMobiSim. This program is based off of the CanuMobiSim architecture. VanetMobiSim possesses the capability to modify both macro and micro-mobility factors when creating and running different traffic scenarios. The hope is with proper attention to all factors that affect traffic conditions, a model can be developed that more accurately depict real world traffic situations than previous models.

### VanetMobiSim

As stated, VanetMobiSim is derived from CanuMobiSim. The advantage of modeling with this program is that in addition to basic modeling criteria it allows for the user to fine tune a simulation by configuring many macro and micro-mobility settings.

Macro-mobility features pertain to physical conditions that influence traffic. This includes physical characteristics of roads such as the number of lanes, whether a road has 2-way traffic, the speed limit, etc… What is also included with VanetMobiSim is the ability to detail road topologies. Road topologies are important because they also play an important role in shaping the characteristics of traffic. In VanetMobiSim they are implanted as graphs where the edges determine the path of vehicles. There are a few types that can be implemented which are briefly explained with an example. Both User-defined topology and GDF map topology are defined in CanuMobiSim as well.

| Various Supported Road Topologies |  |
| --- | --- |
|  | User-defined graph: “the road topology is specified by listing the vertices of the graph and their interconnecting edges.” [2] |
|  | GDF map: “the road topology is imported from a Geographical Data File (GDF). Unfortunately, most GDF file libraries are not freely accessible.” [2] |
|  | TIGER map: “the road topology is extracted from a map obtained from the TIGER database. The level of detail of the maps in the TIGER database is not as high as that provided by the GDF standard, but this database is open and contains digital descriptions of wide urban and rural areas of all districts of the United States.” [2] |
|  | Clustered Voronoi graph: “the road topology is randomly generated yb creating a Voronoi tessellation on a set of non-uniformly distributed points. This approach is similar to that proposed in [10], but we also consider the presence of areas with different road densities which we refer to as clusters.” [2] |

Some of the benefits of VanetMobiSim are the additions of certain types of macro-mobility characteristics. Added features include modeling of roads with 2-way traffic with multiple lanes, the option of having road dividers, varying speed limits, and the introduction of different traffic signs at points of interest. The user has the ability to place stop signs or traffic lights at varying intersections to further enhance the accuracy of the simulation.

VanetMobiSim also includes the ability to incorporate both a trip generation module and a path computation module. The first module allows for a user to set multiple points of interest for a vehicle to follow while the second module uses an algorithm to determine the best path between two points.

With the trip generation module there are two viable options available for the simulation. Given road topology there is the option to implement a random trip model. This sets two points of interest on the map and the simulation will take random paths along the edges of the graph to reach the destination point. The other option is to implement the activity sequences generation where the user will define the two points of interest and explicitly define the edges to travel in which the vehicles will follow.

The path computation module allows for a decision to be made on which path to take based on some criteria of a given road topology. In this simulation there are three possible path computations that are available for use. The general method is by applying a weight with the edges of the graph. In the first method the shortest path is selected using a form of Dijkstra’s algorithm. The cost increases as the length of an edge decreases. The second method applies a cost to an edge based on the number of vehicles on the road. The intent is to mimic a live setting with varying vehicle densities. The last method is a combination of the first two methods with the caveat of having knowledge of the speed limit of roads. With this information a decision can be made based on the fastest clearest paths to reach a desired destination.

The micro-mobility features pertain to the actions of an individual car. This is arguably one of the most important features to VanetMobiSim in generating realistic simulation results. The ability to vary how individual vehicles move and accelerate is the intangible that is missing from many simulation programs. To infuse the desire to simply pass a slower vehicle into the simulation may not seem like much, but it is a factor in the real world and is important to characterize traffic conditions realistically.

With VanetMobiSim there are three classes of micro-mobility features. The three features are classified based on the how the acceleration patterns are calculated. The acceleration classes can be deterministic, as a function of nearby vehicles in a single lane setting, or as a function of nearby vehicles based on multiple lane and flow traffic conditions.

With CanuMobiSim it was particularly good at modeling random actions of single vehicles. The weakness lies with its ability to model groups of vehicles. Although it could model some random actions, the paths of motion ignored other vehicles; therefore it is possible for two vehicles to overlap. Obviously this could never happen in the real world as car accidents are the result. Where VanetMobiSim improved is the ability to simulate using the The Fluid Traffic Model (FTM) or Intelligent Driver Model (IDM). These two models are based on single lane models and do not take into account multiple lanes.

| Fluid Traffic Model (FTM) |  |
| --- | --- |
| s=maxsmin,smax 1- kkjam | This is a monotonically decreasing equation with a lower bound as a result of traffic congestion. - s: output speed - smin and smax are minimum and maximum speed respectively - kjam: vehicular density when a traffic jam is detected - k: current vehicular density of the road. Defined as k= nl where n is the number of vehicles on the road and l is the length of the road. |

| Intelligent Driver Model (IDM) |  |
| --- | --- |
| dvdt=a1-vvo4-s*s2 Where s*=so+vT+v∆v2ab | IDM models the behavior of a vehicle based on the vehicle in front of it. This makes it part of the well known car following model. - dvdt: Instantaneous acceleration - v: current speed of the vehicle - vo: desired velocity - s: distance from preceding vehicle - s*: desired dynamical distance. A function of the minimum bumper to bumper distance so. - T: minimum safe time headway - ∆v: speed difference with respect to front velocity - a: maximum acceleration - b: maximum deceleration |

A micro-mobility feature added in VanetMobiSim is the ability to handle different scenarios at intersections. This new model is referred to as the Intelligent Driver Model with Intersection Management (IDM-IM). IDM-IM has the capability to simulate intersections with either a stop sign or traffic light. Although it seems trivial, the stop and go associated with a stop sign and the continuous flow and extended stops with traffic lights affect traffic not only at the current intersections but also intersections and roads in its vicinity. In the case when a vehicle is approaching an intersection with a stop sign it is modeled with the following parameters:

| Stop Sign: When a vehicle reaches the stop sign it is passed information about how many cars are currently waiting for their turn to go at the intersection. It waits its turn to move following the right hand rule policy just like we learned in traffic school. |  |
| --- | --- |
| s= σ-S ∆v=v | σ: current distance to the intersection S: safety margin for where the vehicle should stop and the center of the intersection |

The second feature handled by IDM-IM is when a vehicle is approaching an intersection with traffic lights. If the approaching intersection has a traffic light in the state RED, then the vehicle begins to decelerate as it approaches the intersection. If the state is GREEN then the vehicle maintains its traveling velocity through the intersection. Another intersection feature is the ability to accelerate or decelerate based on a change in state of the traffic signals. If a vehicle is decelerating towards an intersection that has a current state of RED light, but before it reaches a complete stop the light changes state to GREEN, then the vehicle will begin to accelerate, up to a maximum velocity, through the intersection. The opposite is true if the state of the light changes from GREEN to RED. The minimum breaking distance for the traffic light scenario is described:

| Minimum Break Distance s |  |
| --- | --- |
| s=vt- kb2t2 let t= vkb =vvkb-kb2vkb2 s= v22kb | Given the current speed of a vehicle, the minimum breaking distance characterizes the distance needed to come to a complete stop. - s: minimum breaking distance - v: Vehicles’ speed - t: time - kb: deceleration value |

If the minimum breaking distance is smaller than the necessary distance needed to stop before reaching the intersection, s> σ-S, then the vehicle will move through the intersection even if the state of the traffic light is RED. I believe this accurately represents real situations in which sometimes, as a driver, the traffic light changes to RED and I know that I cannot safely stop. In that situation there is no choice really but to go through the intersection, which is what VanetMobiSim mimics in this micro-mobility feature.

The other significant addition for micro-mobility features introduced in VanetMobiSim is Intelligent Driver Model with Lane Changes (IDM-LC). This feature allows for the user to simulate factors in which a vehicle decides to change lanes or, in some instances, change lanes to pass a slower leading vehicle. Of course, we know that changing lanes cannot happen at any time given the possibility of collisions. The simulator takes into account the available roads that it can change to and uses macro-mobility information to determine whether it is safe to change lanes. In the event that a vehicle needs to change lanes, it can only do so if the circumstances lead to a safe transition. Otherwise, it must adjust its speed until there is an opening to change lanes.

Addressing the decision to change lanes, VanetMobiSim incorporates the MOBIL model to determine when a vehicle should change lanes to overtake another vehicle. The basis of this protocol relies on the minimization of vehicle braking. If the simulator calculates that there will be less braking by changing lanes then a lane change to overtake a vehicle occurs. The protocol is described by the following:

| MOBIL Braking Model Conditions |  |
| --- | --- |
| al- a + abias>p acur+anew- acurl-anewl+ atrh anewl>-asafe | - l: lane to change to - a: acceleration - p: politeness factor - athr: threshold acceleration, minimum needed to allow a lane change - abias: adds favor to moving to a particular side, in this case positive for right and negative for left - asafe:new back vehicle deceleration must be greater than this threshold |

The first equality weighs the positives (left side) of changing lanes against the negatives (right side) of the other vehicles in the current and new lane. If the positives of changing outweighs the disadvantages of the other vehicles in the new and current lane then the first criteria is met. Note that there is a politeness factor, p, which is incorporated into the model. The higher the value of p the more polite the driver is to vehicles in the current and new lane. For small values of p, this represents a driver that drives in a selfish manner and will change lanes more frequently. The second equality represents the acceleration of the now following vehicle. If it can safely brake, the value is given by asafe, then we now meet both criteria and the lane change will occur.

## Simulation

Figure 3 shows the road topology that defines the road layout for the simulations. This represents a city of 1500m by 1500m where the thin lines represent roads with a speed limit of 15 m/s (54 km/h) and the thick lines have a speed limit of 20 m/s (72 km/hr). A circle or square on the road topology represents entry and exit points of vehicles for the simulation. The difference is that circles represent entry and exit points of the higher speed vehicles while the square represents the lower speed vehicles. The vehicles will travel according to activity sequences, where described earlier the vehicles will travel along determined edges of the road. Also in figure 3 the transition probability matrix is represented as an activity chain. It also has the square representing slower vehicles and circles representing faster vehicles.

The traffic density for this simulation varies from 10 vehicles/km to 50 vehicles/km. Data was collected for 3600s with a different simulation for each type of modeling. In figure 4 we can see the results of the simulation and the affects of traffic density to the average speed of vehicles. In the following tables list the parameters that were used for the simulation given each traffic model.

| Model | RWP |  | CSM |  | FTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Speed | Pause | Speed | Pause | smin | smax | kjam |
| Value | unif10,20ms | unif0,60s | unif10,20ms | unif0,45s | 3ms | 20ms | 0.125carm |

| Model | IDM |  |  |  |  | IDM-IM | IDM-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | vo | so | T | a | b | k | abias | p | athr |
| Value | unif10,20ms | 1 m | 0.5 s | 0.6ms2 | 0.9ms2 | 5 | 0.2ms2 | 0.5 | 0.2ms2 |

## Performance Results

Looking at the results for RWP (Random Way Points) and CSM (Constant Speed Motion) we can see the importance of a realistic model. The results show that for both RWP and CSM, as traffic density increases there is no loss in average speed. For anyone that has been on a highly populated city road during rush hour, we are assured that this is definitely not the case. By not taking into account car to car interactions on the road both models conclude that an average speed is attained no matter the traffic density.

### FTM (Fluid Traffic Model) Results

Examining figure 4 the FTM has the highest average velocity in low and high vehicular density scenarios. This is partly due to the parameters of the model where vehicles are never traveling at 0 m/s otherwise the model would break. In any case, the FTM shows a decrease in velocity as vehicular density increases, which is expected. By having each vehicle react to the surrounding vehicles around it, we know that increased traffic density, such as rush hour traffic, will lower the average speed of vehicles. However, a weakness with the FTM is that it doesn’t properly model the vehicular densities at intersections as it doesn’t differentiate the straightaway paths from intersections as seen in figure 5.

Going back to figure 3, which defines the road topology, we see there is a massive increase in traffic density at points A and B and all other roads have approximately the same density. What this translates to is that A and B is a highly travelled road, however once past that section it is as if traffic instantly vanishes; an unrealistic situation. Also, since this model does not account for stoppage at intersections we can see a smooth transition across intersections which are also another weakness with FTM.

### IDM (Intelligent Driver Model)

IDM reacts to the vehicle in front of it, which classifies it as a form of the car following model, so understandably it is affected by traffic density. From figure 4 it is apparent that as the density increases the average speed of vehicles decrease. The desired dynamical distance, s*, in this model has affects both on the velocity and distribution of density on the road topology. As more vehicles enter the system the model tries to keep the desired dynamical distance, as a consequence as the vehicular density increases the model slows the speed of the vehicles. However one drawback to this modeling constraint is lack of intersection modeling. Looking at figure 6 we can see extremely high densities at the most frequented intersections.

From the road topology in figure 3 and looking at intersections A, B, and C in figure 6 there is a spike in vehicular density. This is due to the desired dynamic distance modeling and its affects at intersections. Because every vehicle attempts to maintain a minimum distance from other vehicles a bottle neck of sorts is created at intersections. When an oncoming vehicle is detected a vehicle is forced to slow down or in some instances stop until its path is clear. This clearly causes the increased density results seen at intersections A, B, and C. Since there is no intersection modeling it is unpredictable how long each vehicle will queue up at intersections. Although the average speed is realistic, the density chart shows that this model is still lacking.

### IDM-IM Stops and Lights (Intelligent Driver Model with Intersection Modeling)

With IDM-IM for both stop signs and traffic lights we can see, in figure 4, that the average velocity is noticeably lower than the original IDM case. This is directly related to the vehicles being forced to stop at intersections whether by a stop sign or traffic light. We can also see that the results, in the higher density vehicle scenario, with stop signs have a slightly lower average velocity because every vehicle is required to stop at intersections whereas in the case of traffic signals it is possible for cars to go through intersections without stopping if the lights are GREEN. This is not true for low density situations because when roads are relatively empty, vehicles are able to move through intersections as the first vehicle waiting for a stop sign. In the traffic signal case if a vehicle were to get caught at a RED traffic signal, it would still have to wait for the signal to become green before it can proceed to move through the intersection.

In figures 7 and 8 there is a better distribution of vehicles at every intersection, giving a more realistic simulation. For intersections A, B, and C in both figures (High traffic intersections) we also see that the density gradually increases as a vehicle approaches an intersection, representing queuing of vehicles when reaching intersections. Analyzing figure 7, the stop sign case, we now can see that there is a slight density build up at all intersections. Particularly at intersections A, B, C, and D we see a spike in the density because these are heavily travelled roads. With a stop sign we also see lower density spikes at intersections that are not heavily traveled. Compared with figure 8 the density spikes at intersections A, B, C, and D are smaller because traffic signals allow for bursts of vehicles to move through the intersection as opposed to one by one. Clearly this mechanism is superior in busy intersections as the traffic signal protocol tempers the density at those intersections. However at all other intersections that are not heavily traveled, it appears that the traffic signals produce a higher vehicle density. As stated earlier, there will be vehicles that must stop at intersections even if there is no cross traffic. The results suggest that for busy intersections a traffic signal protocol should be implemented while stop signs should be used to control less frequented streets.

### IDM-LC Lights (Intelligent Driver Model

The last simulation incorporates all the micro-mobility features discussed thus far; Intelligent Driver Model, Intersection Management (Lights in this case), and Lane Change Modeling. First looking at figure 4 we see that as traffic density increases the average velocity only slightly decreases. A reasonable explanation for this result is having the ability to change lanes. The first reason is that there was more than one lane, effectively aiding in reducing the density per lane. As we have soon throughout all the simulations are that a lower density of vehicles correlates to a higher average velocity. The second reason is the ability to avoid the queuing effect by changing lanes to avoid slower vehicles. If we compare figure 8 and 9 paying attention to intersections A, B, and C we can see that by allowing vehicles to change lanes the density spikes occurred closer to the intersections. In figure 8 we see a more gradual build up of density, due to the queuing effect, where as in figure 9 the density spikes are more located in the vicinity of the intersections. Also looking at the other intersections that are not as busy, we see the same distribution of density as in figure 9. In those intersections since there is not much contention with slower vehicles, the density is a function of the traffic lights timing mechanisms.

## Possible Improvements on the Model

## Conclusion

In this conference article we were introduced to a very useful traffic simulation tool VanetMobiSim. We clearly saw the importance of being able to control macro and micro-mobility features as which was evident in the simulation results of multiple types of models. The importance of a proper traffic simulation model cannot be overstated because it is an essential tool to further propel the ever advancing VANET technologies. The resources needed to conduct real life experiments are extremely costly given the parameters we set in the simulations. To model traffic density alone would require extreme coordination and monetary resources.

# Analytical Model for Connectivity in Vehicular Ad Hoc Networks

# Authors: Saleh Yousefi, Eitan Altman, Rachid El-Azouzi, Mahmood Fathy

# IEEE Transactions on Vehicular Technology, Vol. 57, No. 6, November 2008

## Describe Problem

In this paper the effects of vehicular speed is modeled and analyzed to understand the consequences it has on VANET connectivity. Tools from traffic and queuing theory are used to develop models for the distribution of vehicles and connectivity to better understand how speed is a factor. Then through multiple simulations the validity of these models will be verified.

We begin detailing the use of the “fundamental traffic theory equation. [3]” This equation gives a general relationship between the macroscopic parameters of speed, vehicle density, and vehicle flow as described:

| Fundamental Traffic Theory Equation |  |
| --- | --- |
| F=S×K | - F: Traffic flow - S: Average speed - K: Traffic Density |

This equation, when plotted, forms two distinct regions for the traffic flow. When traffic density is relatively low vehicles are free to move and never form a queue which is deemed stable flow. In this region at very low density vehicles are in a state called free flow because they can move freely and at any speeds. However as density increases the traffic flow eventually begin to degrade which is the forced flow section of the graph. The point in which this phenomenon occurs is called the critical value. Figure 1 details the various regions.

Forced Flow

Stable Flow

Notice that the density can increase so much, representing a traffic jam, to the point where no vehicles are moving.

The focus of this article will be when the system is in a stable flow state. Because we are dealing with VANET communications, the issue of connectivity is something that must be taken to account. From the nature of VANET’s, the number of vehicles in the system affects the connectivity of the network. The denser a VANET network is the better the connectivity. For this reason the emphasis will be in the stable flow setting to create an analytical framework when there can be discontinuity in the network.

The model used in this article consists of a point on a highway that records metrics of vehicles passing. From the data set is generated by vehicles with a random exponential distribution when they pass this point which can be used to formulate the distribution of distances between vehicles. Using these parameters a framework is formed and will be validated through simulations. Also, a more in depth analysis will be done on the effects of vehicular speed on network connectivity by employing stochastic ordering techniques. This will be done by leveraging the concept that VANET connectivity due to distance and speed is similar to an infinite server queue during busy periods.

## Analytical Model

A VANET’s connectivity distance distribution relates to the busy period of an infinite server queuing system and the number of customers served in that period has a similar distribution to the number of vehicles connected in a VANET system. The inter-arrival times in an infinite server queue will serve as the distribution for the inter-vehicle distance between each successive node in a VANET. The last component to make this relation relevant is having the service time distribution relate to the transmission range of the VANET.

### Distribution of Inter-vehicle Distances

To develop the distribution for the inter-vehicle distance we define the setup as having an observer at a location on a highway that monitors vehicles that pass. We assume that the vehicles pass the observer with a Poisson distribution and we know the expectation of a Poisson distribution is 1λ. Given this we know that the flow of traffic is described by λ and we can conclude that the inter-arrival times are exponentially distributed by parameter λ. Let us also define the velocity of vehicles as vi i=1,…,N where N will be the number if discrete states of velocity. By having levels of discrete speeds we also affect the arrival rate of vehicles into the system so that λi(i=1,…,N) where i=1Nλi=λ. Given this definition then the probability of a vehicle arriving at a particular speed is given by Pi=λiλ.

To help facilitate the development of the framework we introduce for definitions:

- tni is the time when the nth car (n > 1) of speed vi arrives to the observer

- Tni=tni-tn-1i where t0i=0. Define {Tni} as the inter-arrival time sequence of vehicles with speed vi exponentially distributed where the expectation ETni=1λi

- Sni= Tnivi Define Sni as the inter-vehicle distance sequence that is exponentially distributed and the expectation is ESni= vi/λi. Let us also define Rnj≔ Snj

- Ln is the distance between the nth closest car to the observer and (n-1)th closest car to the observer at time 0.

In figure 2 we see an example of an observer with vehicles travelling at two discrete speeds. The S distances are of each successive vehicle travelling at the same velocity. The R distances are a combination of vehicles travelling at the same and different speeds. The L distances are the successive minimum of R speeds between each vehicle regardless of the speeds. Since the L distances are the essentially the minimum of R distances, and we stated earlier that the R distances are equal to the S distances, it reasons that the L distances are also exponentially distributed.  In order to define the distribution for Ln we define the following:

- Rnj≔ m=1kn-1jSmj- k=1n-1Lk, j≠αn-1Rnj≔Skn-1jj, j=α(n-1)

- αn:= arg mini Rni represents the speed index of the car that is closest to the observer at time 0, excluding the first (n-1) closest vehicle.

- knj≔1+ l=1nIαl(j) where  l=1nIαl(j) is the number of cars with speed index j among the nth closest car to the observer and Iαl(j) is the indicator function; it is equal to 1 when l=j and zero otherwise.

From general studies of probability we know that an exponential function has the memory less property and that each L distance is independently and identically distributed. From this we can deduce that PL2>t = P (L2>L1+t|L2>L1). Since we know that each R distance is distributed by parameter λjvj and each L distance is the minimum R distance for each speed index, then we can define:

PL>x= 1- FLx=e-i=1Nλivix=e-λi=1NPivix

Going back to figure 2, we can see how the following example illustrates how to calculate each L inter-vehicle distance.

- R11=S11,  R12=S12                                        →L1=minR11,R12

- R21=S21,  R22= S12-L1                                         → L2=min⁡(R21,R22)

- R31=S11+S21-L1-L2 , R32=S22              →L3=minR31,R32

- R41=S11+S21-L1-L2-L3,  R42=S32→L4=min⁡(R41,R42)

To help define parameters Pi and vi, where Pi=λiλ,  we leverage many observations in traffic theory that shows free flow traffic have a normal distribution. With this assumption and given experimental values listed in Table I, we can formulate the probability density function of the speed as follows with μ as the mean and σ as the standard deviation:

fvv= 1σ2πe-(v-μ)22σ2

Table I: Normal-Speed Statistics

| μ[km/h] | σ[km/h] |
| --- | --- |
| 70 | 21 |
| 90 | 27 |
| 110 | 33 |
| 130 | 39 |
| 150 | 45 |

To make the model realistic a maximum and minimum bound (vmax and vmin) are enforced. If we also define the function,

fvv= fvvvminvmaxfvsds=2fv(v)erfvmax-μσ2- erf⁡(vmin-μσ2)

Where erf(…) is the standard error function, then we can describe the occurrence probability of each speed as:

PL>x= e-λxvminvmaxfv(v)vdv where vmin<v< vmax

Determining vmin and vmax is the last piece needed to characterize the occurrence probability function. We know that the given a normal distribution function 99.7% of an event occurring lies between (μ-3σ, μ+3σ). This boundary is sufficient to characterize almost all the traffic flow in derived model, therefore we can set vmin=μ-3σ and vmax=μ+3σ.

### Connectivity Analysis

There are two important metrics to evaluate when looking at the connectivity of a VANET system. The first is the cluster (platoon) size of a connected system. This is the number of vehicles, or nodes, that are currently connected. This is important because the platoon size essentially states how many vehicles are able to receive messages that are transmitted in the VANET system. The second is connectivity distance, which is essentially the area which the network is connected.  This metric is important because it details the space in which a vehicle can enter a VANET system and receive broadcast messages.

To develop an expression for the platoon size we revisit the relation of a busy period in a G/D/∞ queuing system . By relating the number of customers served with the number of vehicles in a platoon and taking the Z transform of the probability generating function of the queuing system, we arrive at the probability generating function of the platoon size to be:

Nz= z1- FLR1-zFLR

where FL.is the itervehicle distribution and R is the fixed transmission range

Performing an inverse Z transform we can find the probability mass function (pmf) of the platoon size: PN=k=1-FLR∙FL(R)k-1,  k=1,2,3…

Now we can form an expression for the probability that at least k vehicles are connected also referred to as the tail probability of the platoon size. This is simply the summation of the pmf of the platoon size beginning at k.

PNk= PN≥k=t=k∞PN=t=FLRk-1
=1-e-iλiviRk-1
=1-e-λRE(1V)k-1

By taking the expectation of the probability generating function of the platoon size as the limit of z approaches 1 we can derive an expression for the average number of vehicles in a platoons.

EN=limz→1∂N(z)∂z=11-FL(R)=1e-iλiviR=1e-λRE(1V)

In terms of the number of relays of a message, or the number of hops, required to transmit a message from a source to destination vehicle depends on the routing protocol, position of the destination of vehicles, etc.. In any case if we denote the number of hops as Nhops, we know that given any protocol Nhops≤N-1 . Given any situation there would not be more hops then there are number of vehicles. With this upper bound constraint in mind we can say that Nhops≤EN-1 . We can now leverage this constraint to find the upper bound of the tail probability of the number of hops with the following derivation:

PNhops≥h≤PN-1≥h
PNhops≥h≤PN≥h+1
PNhops≥h≤1-e-λRE(1V)h

The connectivity distance can be derived beginning, once again, from the Laplace transform of the busy period of a G/D/∞ queuing system. Let d be a random variable to represent the connectivity distance and fd(s) be the Laplace transform, then:

fds=e-sR1-FLR1-0Re-sxdFLx
=(s+iλivi)e-s+iλiviRs+(iλivi)e-s+iλiviR

Given this expression, we can take the negative derivative of fds with respect to s and evaluating at s=0 to find the average connectivity distance.

Ed= -∂fds∂ss=0=1-e-iλiviRiλivie-(iλivi)R
=1-e-λRE(1V)λE(1V)e-λRE(1V)

## Simulation and Analysis

Simulations were conducted to evaluate the validity of the models derived earlier. Because the focus is during the free-flow state of traffic, there is no need to use advanced traffic simulation tools that incorporate microscopic factors. MATLAB is sufficient to conduct the simulation, having a 20km uninterrupted highway with vehicles entering the system with a Poisson distribution with parameter of 500 veh/h. Vehicles entering the system will have speeds of 80, 110, 130, and 180 km/h with a probability of 20%, 30%, 30%, and 20% respectively.

Figure 3 shows the results of the tail probability of the inter-vehicle distance. The simulation ran 100 times with different seed values and the “x” represents the 95% confidence interval. This holds true for each simulation plot here after. Looking at the plot, the results from the model lie comfortably within the confidence interval. Also, the shape of the curve fits an exponential curve and the simulated mean was calculate to be 231.95 and the theoretical mean was 231.32, well within the 95% confidence interval.

Fig. 4. Comparison of the simulation measurement and the model prediction for (a) the tail probability of the platoon size and (b) the average platoon size.

In figure 4(a) the plot shows the tail probability of the platoon size, in other words the probability that at least k vehicles are connected. Given that the traffic flow is 500 veh/h, representing rather sparse traffic, we expect the probability of having at least k vehicles connected in the VANET to decrease as k increases. With a low density of vehicles it is difficult to have a high number of vehicles within close enough range to be connected together. This leads to the results in figure 4(b) that as traffic flow increases, corresponds to an increase in traffic density, that the average platoon size increases. Obviously as more vehicles are in range and connected, the direct result is that the average platoon size increases as well.

Fig. 5. Comparison of the simulation measurement and the model prediction for (a) the tail probability of the connectivity distance and (b) the average connectivity distance.

Figure 5 are the results of the model plotted against 3000 simulations runs for the tail probability of connectivity distance. In figure 5(a) the tail probability of connectivity distance is plotted against vehicle distance. What the simulation, as well as the framework presented for  Ed, shows is that as the distance increases between subsequent vehicles that the average probability of being connected decreases. This is intuitive given the transmission range and traffic flow because as distance increases more vehicles fall out of transmission range. For this reason, figure 5(b) makes intuitive sense because as traffic flow increases, which increases traffic density, the average connectivity distance also increases. With the increased density of vehicles there are more chances for more vehicles to be connected thus increasing the average connectivity distance.

### Varying Traffic Parameters

We saw how the changes in average distance between vehicles and traffic flow influenced the connectivity in a VANET system. Now we will investigate the ramifications when varying the transmission ranges, the average value and standard deviation in the normal distribution, and again the traffic flow to uncover the intricacies of the relationship between all these parameters.

Fig. 7. Effect of the vehicle’s transmission range on (a) the tail probability of the platoon size and (b) the average platoon size.

Re-visiting the tail probability of platoon size,  PNk=1-e-λRE(1V)k-1, and the expected number of vehicles in a platoon, EN=1e-λRE(1V), we analyze the results when varying the transmission range. Looking at figure 7(a) we first see that as the transmission range of the communication devices increase so does the tail probability of the platoon size. Intuitively this is what we’d expect because given a greater broadcast range there will be more vehicles within range of forming the VANET. An example is the 50% mark of the extremes where for a range of 200m the tail probability of the platoon size yields 5 vehicles as opposed to the range of 800m which yields nearly 30 vehicles. With the conclusion that the tail probability increases with transmission range, then the next logical assumption is that the average size of platoons would also increase because there is a higher probability of having increased vehicles in a platoon. As seen in figure 7(b) this is the exact results from the simulation and the analytical model. Furthermore, we see that the increase in average platoon size is exponential, so the benefits of increasing transmission range from 500m to 800m is quite significant.

Fig. 8. Effect of the vehicle’s transmission range on (a) the tail probability of the connectivity distance and (b) the average connectivity distance.

Figure 8 shows the effect of transmission range on the tail probability of connectivity distance. In a similar vein as the platoon size, as we increase the range we see the connectivity distance probability increase. The increased probability leads to an exponential increased average connectivity distance. The platoon size and connectivity distance are tightly coupled as more vehicles inevitable stretch the bounds of a VANET thus increasing the connectivity distance.

Fig. 9. Effect of traffic flow on (a) the probability of the platoon size and (b) the average platoon size.

The effects of a varying traffic flow are very similar to increasing the transmission range.  The mathematical models for tail probability of the platoon size and average platoon size also increased with increased traffic flow. The reasoning is similar to increasing the transmission range, with an increased vehicle flow the density of vehicles increases. With an increased vehicle density more vehicles are within the transmission range of communication devices. The increased vehicles create a larger platoon which is reflected in figure 9. The other parameter affected is the connectivity distance which also increases with traffic flow. As the platoon size increases the adverse consequence is that the VANET physically increases thus increasing the connectivity distance.

## Performance Results

## Discuss Results

## Suggestions

# References

## [1] Routing in Sparse Vehicular Ad Hoc Wireless Networks

Authors: Nawaporn Wisitpongphan, Fan Bai, Priyantha Mudalige, Varsha Sadekar, and Ozan Tonguz

Publication: IEEE Journal on selected areas in communications, Vol. 25, No. 8 October 2007 (pg 1538-1556)

## [2] Vehicular Mobility Simulation for VANETs

Authors: Marco Fiore, Jerome Harri, Fethi Filali, Christian Bonnet

Publication: Conference – Simulation Symposium, 2007. ANSS ’07. 40th Annual

## [3] Analytical Model for Connectivity in Vehicular Ad Hoc Networks

Authors: Saleh Yousefi, Eitan Altman, Rachid El-Azouzi, Mahmood Fathy

Publication: IEEE Transactions on Vehicular Technology, Vol. 57, No. 6, November 2008